The Bombay Salesian Society’s

DON BOSCO INSTITUTE OF TECHNOLOGY

An Autonomous Institute Affiliated to University of Mumbai, Approved by AICTE, NAAC ”A” Grade

Premier Automobiles Road, Kurla (W), Mumbai-70

# Affiliated to the University of Mumbai

# 25CE4CEP02 Community Engagement Project

# S.E. MINI PROJECT REPORT ( SEM 4)

On

“ Train Overcrowd Detector: TrainSthiti”

Atharv Dalvi– Roll No. 22

Roger Dsouza – Roll No. 35

Rahul Dubey – Roll No. 38

Darren Fernandes – Roll No. 40

Department of Computer Engineering      25-26

May 2026

# CERTIFICATE

Project Title 	:  Train Overcrowd Detector: TrainSthiti

Organization 	: Don Bosco Institute of Technology

Address 	: Premier Automobiles Road,

Kurla (W), Mumbai-70

Project Team Members 	: 	  Atharv Dalvi– Roll No. 22

Roger Dsouza – Roll No. 35

Rahul Dubey – Roll No. 38

Darren Fernandes – Roll No. 40

Project Co-Ordinator 	:     Ms. Pooja Bansode

EXAMINER 1 	EXAMINER 2

HEAD, COMPUTER ENGINEERING

Department of Computer Engineering

Don Bosco Institute of Technology, Mumbai – 70

ABSTRACT

Mumbai’s suburban railway system is one of the busiest public transportation networks in the world, carrying millions of passengers every day. Due to the absence of a real-time crowd monitoring system, commuters often face unsafe and uncomfortable travel conditions caused by overcrowded train coaches. Existing railway applications only provide train schedules and route information but fail to provide live crowd density data.

The proposed project, “TrainSthiti – Train Overcrowd Detector,” aims to solve this problem by developing an IoT-based crowd monitoring and notification system. The system uses Raspberry Pi devices equipped with Wi-Fi and Bluetooth scanning capabilities to estimate passenger density by detecting nearby mobile devices. The collected data is sent to a backend server where it is processed, classified into crowd levels, and displayed through a user-friendly web or mobile interface.

The project integrates IoT devices, backend APIs, databases, and frontend applications to provide a complete smart transportation solution. The system is designed to be scalable, cost-effective, privacy-friendly, and suitable for deployment in India’s railway infrastructure. TrainSthiti has the potential to improve commuter safety, reduce overcrowding risks, assist railway authorities in crowd management, and provide better travel planning for passengers.

INTRODUCTION

Mumbai local trains are considered the lifeline of Mumbai and are used daily by more than 8 million commuters. Despite being one of the most important public transportation systems in India, the railway network continues to face major overcrowding issues, especially during peak hours.

Currently, passengers have no reliable way to determine how crowded a train coach is before boarding. As a result, commuters are forced to rely on assumptions or manual observation, which often leads to dangerous situations such as pushing, overcrowding, and accidents during boarding and deboarding.With the advancement of IoT, cloud computing, and real-time data analytics, it has become possible to develop intelligent systems capable of solving transportation-related problems efficiently. TrainSthiti is designed as a smart crowd monitoring solution that combines hardware and software technologies to detect and display real-time crowd density inside train coaches.

The system uses Raspberry Pi hardware to scan Wi-Fi and Bluetooth signals emitted by nearby devices. These signals are processed to estimate crowd density and provide live updates to commuters and railway authorities. By integrating cloud databases, APIs, and frontend applications, the project provides a complete and scalable smart transport solution.This project demonstrates how modern technologies can be used for public welfare and community engagement while addressing a major real-world problem faced by millions of people every day.

PROBLEM STATEMENT

Mumbai’s local train system currently lacks a reliable real-time crowd monitoring solution. Commuters board trains without knowing the crowd density inside coaches, leading to unsafe and uncomfortable travel conditions.Existing railway applications only provide train schedules, timing, and route information but fail to provide real-time crowd updates.Therefore, there is a strong need for a smart and automated system capable of detecting and displaying live crowd density inside train coaches.

OBJECTIVES

The primary objective of TrainSthiti is to develop a smart crowd detection and monitoring system for Mumbai local trains.

Specific objectives include:

- To develop a system capable of detecting crowd density using IoT-based device scanning

- To reduce overcrowding-related accidents and improve passenger safety

- To provide real-time crowd information through a mobile or web application

- To improve travel planning for commuters by displaying crowd status before boarding

- To assist railway authorities in crowd management and operational planning

- To create a low-cost, scalable, and privacy-friendly transportation solution

METHODOLOGY

The TrainSthiti system follows a structured methodology involving hardware scanning, data processing, backend communication, and frontend visualization.

Step 1: Signal Scanning

A Raspberry Pi device installed inside train coaches continuously scans nearby Wi-Fi and Bluetooth signals emitted by passenger smartphones.

Step 2: Device Detection and Filtering

The system identifies unique devices and filters duplicate or irrelevant signals using signal strength analysis.

Step 3: Data Processing

The detected device count is processed and classified into crowd levels such as:

- Low Crowd

- Medium Crowd

- High Crowd

- Overcrowded

Step 4: API Communication

The processed data is sent to the backend server through REST APIs.

Step 5: Database Storage

Crowd data is stored in databases such as MongoDB or PostgreSQL for real-time and historical analysis.

Step 6: Frontend Display

The frontend application displays live crowd information using color-coded indicators and train details.

Working Flow

Wi-Fi/Bluetooth Scan → Device Detection → Crowd Classification → API Communication → Database Storage → User Interface Display

TARGET COMMUNITY

The primary target community for TrainSthiti includes daily commuters using Mumbai local trains.

The project specifically benefits:

- Office workers traveling during peak hours

- Students and senior citizens

- Daily railway commuters

- Railway authorities and transport planners

The project also indirectly benefits the larger Mumbai transportation ecosystem by reducing crowd-related risks and improving travel management.

ACTIVITIES CONDUCTED

Several activities were conducted during the development and planning phase of the project.

1. Problem Identification

The team studied overcrowding problems in Mumbai local trains and identified the lack of real-time crowd monitoring systems.

2. Empathy Research and Surveys

Surveys were conducted among commuters to understand crowd-related problems and gather user feedback.

The survey results showed:

- Most commuters face difficulty boarding trains during rush hours

- A large percentage of users expressed interest in a crowd monitoring application

- Daily commuters strongly supported real-time crowd updates

3. Technical Research

Research was conducted on:

- IoT-based crowd detection systems

- Wi-Fi and Bluetooth scanning technologies

- Raspberry Pi hardware capabilities

- Backend and frontend frameworks

4. Prototype Design

A prototype UI was designed to demonstrate how live crowd data would be displayed to users.

5. Technical Stack Selection

The following technologies were selected:

Frontend:

- React.js

Backend:

- Node.js

- Express.js

- Spring Boot

Database:

- MongoDB Atlas

- PostgreSQL

Hardware:

- Raspberry Pi 4

Tools:

- VS Code

- GitHub

- Postman

IMPLEMENTATION

The implementation of TrainSthiti is divided into hardware and software components.

Hardware Implementation

A Raspberry Pi 4 device is used as the main hardware component. It continuously scans nearby Wi-Fi and Bluetooth devices using Linux-based scanning tools.

The hardware setup includes:

- Raspberry Pi 4

- Wi-Fi and Bluetooth modules

- Power supply

- Internet connectivity for API communication

Backend Implementation

The backend server is developed using Node.js and Express.js or Spring Boot.

The backend is responsible for:

- Receiving crowd data from Raspberry Pi devices

- Processing and validating data

- Classifying crowd density levels

- Storing data into databases

- Providing APIs for frontend applications

Database Implementation

MongoDB Atlas and PostgreSQL databases are used for storing crowd information, train details, and historical analytics.

Frontend Implementation

The frontend application is designed using React.js.

The user interface displays:

- Train names and timings

- Live crowd status

- Crowd level indicators

- Train search functionality

The application provides a simple and user-friendly interface for commuters.

TESTING, VALIDATION AND ITERATIVE REFINEMENT

Testing and validation were important phases in the development of TrainSthiti. These stages ensured that the system modules worked correctly, data was processed accurately, and the overall prototype met user expectations.

The testing phase focused on validating the backend APIs, frontend user interface, hardware scanning scripts, and database communication. Iterative refinement was continuously performed based on test outputs and feedback collected during prototype execution.

Unit Testing

Unit testing was performed on individual software modules to ensure proper functionality.

Backend Unit Testing

Backend unit testing was implemented using the Jest framework. Core functionalities tested included:

- Crowd estimation calculations

- Authentication and password validation

- Crowd classification logic

- API payload structure validation

The backend tests ensured that the server correctly processed crowd data and classified density levels based on predefined thresholds.

Frontend Unit Testing

Frontend testing was conducted using Vitest and React Testing Library.

The following frontend functionalities were tested:

- Automatic API polling intervals

- Dashboard updates

- Error handling behavior

- User interface rendering

The frontend tests confirmed that the dashboard refreshed live crowd information continuously without failure.

Manual Script Validation

Manual testing was also conducted on Linux-based scanning scripts such as:

- bluetoothctl

- airmon-ng

- airodump-ng

These tests ensured that raw Wi-Fi and Bluetooth counts were correctly mapped to logical crowd density levels.

Integration Testing

Integration testing was performed to verify communication between different system components.

Backend API Integration

The integration tests verified:

- Data transfer between Raspberry Pi devices and backend APIs

- JSON payload generation and validation

- Database persistence and storage

- API response handling

The generated payloads containing:

- Wi-Fi clients

- Bluetooth devices

- Density levels

- Timestamps

were successfully posted to backend APIs.

Frontend and Backend Integration

Frontend-to-backend integration testing verified that:

- The React dashboard continuously fetched live crowd data

- Historical crowd data endpoints worked correctly

- Dashboard updates occurred approximately every 5 seconds

- Errors were properly handled during API failures

Usability Testing

Usability testing focused on improving user experience and interface clarity.

The dashboard interface was tested to ensure:

- Crowd levels were clearly visible

- Navigation controls were user-friendly

- Mobile responsiveness worked correctly

- Color-coded indicators were easily distinguishable

The visual indicators for crowd density such as:

- Green → Low Crowd

- Yellow → Medium Crowd

- Red → High Crowd

were evaluated for clarity and accessibility.

User Validation and Feedback Analysis

User validation was conducted by comparing observed crowd density with manually counted passengers.

The testing process identified several important observations:

- Device counts sometimes underrepresented actual passenger count because some users kept Wi-Fi or Bluetooth disabled

- Environmental interference affected scanning consistency

- Linux networking services occasionally interfered with scanning tools

Feedback collected during testing helped improve:

- Crowd estimation logic

- API stability

- Dashboard usability

- Error handling mechanisms

Iterative Refinement

The TrainSthiti prototype underwent continuous iterative improvement throughout development.

Several refinements were introduced based on testing feedback:

- Crowd estimation multipliers were recalibrated

- Duplicate device filtering was improved

- Authentication bugs were fixed

- Backend database errors were resolved

- API reliability was enhanced

Execution logs also helped identify critical issues such as:

- Backend communication failures

- BSON circular structure errors

- Network manager interference during scanning

These findings guided the next phase of development and optimization.

Testing Results

The testing phase successfully demonstrated the feasibility of the TrainSthiti system.

The prototype successfully:

- Detected nearby devices

- Processed crowd density data

- Updated the frontend dashboard in real time

- Stored crowd information in the database

- Maintained continuous backend communication

The results confirmed that the system architecture and workflow functioned correctly under testing conditions.

CHALLENGES FACED

Several technical and practical challenges were encountered during the project.

1. Signal Accuracy

The system depends on Wi-Fi and Bluetooth signals being enabled on user devices.

2. Duplicate Device Detection

Filtering duplicate or irrelevant signals was a challenging task.

3. Hardware Limitations

Continuous scanning and processing on Raspberry Pi devices required optimization.

4. Environmental Interference

Crowded railway environments and signal interference affected scanning accuracy.

5. Data Privacy Concerns

Ensuring user privacy while collecting device signals required careful planning.

SOLUTIONS AND IMPROVEMENTS

To overcome the identified challenges, several solutions were proposed and implemented.

- Signal filtering techniques were used to improve scanning accuracy

- Duplicate device removal logic was implemented

- Crowd classification thresholds were optimized

- Only anonymized data was processed to maintain privacy

- Future improvements may include AI-based analytics and sensor fusion techniques

Additional enhancements planned include:

- Integration with official railway applications

- Predictive crowd analysis using machine learning

- Real-time notifications and alerts

- Improved mobile responsiveness and UI design

IMPACT OF THE PROJECT

TrainSthiti can create a significant positive impact on society and public transportation.

Passenger Safety

The system can reduce overcrowding-related injuries and accidents by helping passengers avoid crowded coaches.

Better Travel Planning

Commuters can make informed decisions and select less crowded trains.

Improved Comfort

Passengers can experience safer and more comfortable travel.

Railway Management

Railway authorities can use crowd analytics for:

- Better scheduling

- Resource allocation

- Crowd management

- Operational planning

Smart Transportation Development

The project contributes toward smart city and intelligent transportation initiatives.

FUTURE PLAN

- The future scope of the TrainSthiti project focuses on transforming the current prototype into a fully scalable and intelligent smart transportation solution for Mumbai and other urban railway systems.

- One of the major future goals is to deploy the system across multiple train lines, coaches, and railway stations to provide city-wide real-time crowd monitoring. This would allow commuters to access live crowd information for different trains and routes before planning their journey.

- The project can also be integrated with official railway applications and platforms so that crowd information becomes easily accessible to a larger number of passengers. Features such as real-time notifications and alerts can be added to warn users about overcrowded coaches and suggest safer travel alternatives.

- Future improvements may include the use of Artificial Intelligence and Machine Learning algorithms for predictive crowd analysis. By analyzing historical crowd data, the system can predict rush hours, identify crowd patterns, and provide recommendations for optimal travel timings.

- Additional IoT sensors and technologies may also be integrated to improve accuracy and reliability. The project can further expand beyond local trains and be implemented in metro systems, buses, railway stations, airports, and other public transportation networks.

- In the long term, TrainSthiti has the potential to contribute toward the development of smart city infrastructure by improving public transport safety, operational efficiency, and commuter experience through data-driven decision making.

CONCLUSION

TrainSthiti is an innovative and practical solution developed to address the major issue of overcrowding in Mumbai’s suburban railway system.The project combines IoT, cloud computing, backend systems, databases, and frontend technologies to create a real-time crowd monitoring platform capable of improving commuter safety and travel experience.

By using Raspberry Pi-based Wi-Fi and Bluetooth scanning, the system provides a low-cost and scalable method for estimating crowd density without compromising user privacy.The project successfully demonstrated the integration of hardware and software technologies into a smart transportation solution.

TrainSthiti has the potential to:

- Improve commuter safety

- Reduce overcrowding risks

- Enhance passenger comfort

- Assist railway authorities with operational planning

- Contribute toward smart transportation systems

Although the project is currently in the prototype stage, future improvements and large-scale deployment can make it an impactful solution for urban transportation systems.Overall, TrainSthiti demonstrates how technology and community engagement can work together to solve real-world public problems effectively

REFERENCES

- Aircrack-ng Development Team, “Aircrack-ng Suite Documentation,” Aircrack-ng Official Documentation. Available: Aircrack-ng Official Website
This reference was used for understanding Wi-Fi monitoring tools such as airmon-ng and airodump-ng, which were utilized for detecting nearby Wi-Fi devices and collecting signal data for crowd estimation.

- Raspberry Pi Foundation, “Raspberry Pi 4 Documentation,” Raspberry Pi Official Documentation. Available: Raspberry Pi Official Website
This documentation was referred to for hardware setup, GPIO support, networking capabilities, and configuring Raspberry Pi devices used in the TrainSthiti prototype.

- React Team, “React Documentation,” Meta Platforms Inc. Available: React Official Documentation
This reference provided guidance for developing the frontend user interface of the TrainSthiti system, including component-based architecture, state management, and responsive dashboard design.

- Node.js Foundation, “Node.js Documentation,” OpenJS Foundation. Available: Node.js Official Website
This source was used for backend server development, API integration, asynchronous processing, and communication between the Raspberry Pi devices and cloud database systems.

- MongoDB Inc., “MongoDB Atlas Documentation,” MongoDB Official Documentation. Available: MongoDB Official Website
This documentation was referred to for cloud database setup, data storage, API connectivity, and management of real-time crowd information collected by the TrainSthiti system.Research papers related to IoT crowd monitoring systems

- Mumbai Railway public transportation statistics and reports

ACKNOWLEDGEMENT

We sincerely express our gratitude to our project coordinator, Ms. Pooja Bansode, for her valuable guidance, support, and encouragement throughout the development of this project.

We would also like to thank the Department of Computer Engineering, Don Bosco Institute of Technology, Mumbai, for providing us with the opportunity and resources required to successfully complete this Community Engagement Project.

We are thankful to our friends, classmates, and survey participants for their suggestions and feedback, which helped improve our project.

Finally, we thank all those who directly or indirectly contributed to the successful completion of TrainSthiti.